ASYNCRONIX – THE FIRST DAY

Gait means the way a person walks.

  Leg movement

  Joint angles (hip, knee, ankle)

  Walking speed

  Step length

  Balance and posture

  Pressure on the feet

Why is it important in Exoskeletons?

An exoskeleton must move in sync with the user. To do that, it needs to understand

  When the user starts walking

  When they stop

  Whether they are climbing stairs

  Whether they are sitting or standing

Where does Machine Learning come in?

Sensors collect data from the user:

- Accelerometers

- Gyroscopes

- Force sensors

- EMG sensors (muscle activity)

Machine Learning models are trained on this data to:

- Recognize walking patterns.

- Predict the user's next movement.

- Detect abnormal gait.

- Adjust exoskeleton assistance automatically

1. Upper Body Controls (Most Common)

The user gives commands using:

- Buttons on crutches

- Joysticks

- Smartphone/app controls

- Voice commands

2. Residual Muscle Signals (EMG)

Sometimes a person is paralyzed but still has tiny muscle activity.

Sensors called EMG (Electromyography) sensors detect electrical signals from muscles.

Example:

- User thinks about moving the right leg.

- Small electrical activity appears in the muscle.

- Sensor detects it.

- Exoskeleton moves the right leg.

3. Weight Shifting and Body Posture

The exoskeleton can use:

- Accelerometers

- Gyroscopes

- Pressure sensors

Example:

- User leans forward → Start walking

- User leans left → Turn left

- User leans right → Turn right

4. Brain-Computer Interfaces (BCI) — Advanced Research

This is the futuristic approach.

Sensors on the head detect brain signals using EEG.

Example:

- User thinks "walk"

- Brain signal is detected

- AI model classifies the intention

- Exoskeleton starts walking

1. Accelerometer

Definition

An accelerometer measures linear acceleration along three axes (X, Y, Z).

What it Measures

- Forward/Backward acceleration (X)

- Left/Right acceleration (Y)

- Up/Down acceleration (Z)

Unit

m/s2
2. Gyroscope

Definition

A gyroscope measures angular velocity (rotational speed) around three axes.

What it Measures

- Roll (X-axis rotation)

- Pitch (Y-axis rotation)

- Yaw (Z-axis rotation)

3. Force Sensor

Definition

A force sensor measures force, load, or pressure applied to a surface.

What it Measures

- Ground reaction force

- Foot pressure

- Weight distribution

Unit

Newton(N)
4. EMG Sensor (Electromyography)

Definition

An EMG sensor measures electrical activity produced by muscles.

What it Measures

- Muscle activation signals

- User movement intention

Unit

Usually measured in:

mV
5. Magnetometer

Definition

A magnetometer measures the Earth's magnetic field.

What it Measures

- Direction

- Heading

- Orientation relative to North

Unit

μT
Gait Analysis Perspective

| Sensor | Main Information Obtained |
| --- | --- |
| Accelerometer | Linear movement |
| Gyroscope | Rotational movement |
| Force Sensor | Foot pressure and balance |
| EMG | Muscle activity and intention |
| Magnetometer | Direction and heading |
| IMU | Combined motion and orientation |

Common Algorithms

- Random Forest

- Support Vector Machine (SVM)

- XGBoost

- CNN (Convolutional Neural Networks)

- LSTM (Long Short-Term Memory Networks)

DAY 2 :  JUNE 2 2026

Dataset

↓

Feature Analysis

↓

Data Distribution

↓

Training Time

↓

CPU Usage

↓

RAM Usage

↓

Model Complexity

↓

Accuracy

↓

Deployment Feasibility

1. What is Model Size?

When training finishes:

joblib.dump(model, "Logistic_Regression.pkl")

a file is created.

That file contains:

- learned weights

- learned parameters

- model information

- Why do we care?

- Because in Edge AI:

- Sensor Device
↓
Embedded System
↓
Store Model
↓
Predict Activity

| Model | Size |
| --- | --- |
| Logistic Regression | 3 KB |
| Random Forest | 50 MB |
| Large Neural Network | 500 MB |

We choose LR

2. What is Inference Time?

Training happens once.

Prediction happens thousands of times.

Inference means:

model.predict(X_test)

or in real life:

Sensor Reading
↓
Model
↓
Walking

The time needed for prediction is:

0.002 sec

Why do we care?

Imagine a gait recognition device.

Every second:

Accelerometer
↓
Gyroscope
↓
Model
↓
Activity

If prediction takes:

2 seconds

it's useless.

The person already moved.

Your model predicts in:

0.002 sec

which is excellent.

3. What is RAM Increase?

You measured:

RAM Increase: 0.0195 GB

Convert:

0.0195 × 1024
≈ 20 MB

So training Logistic Regression required roughly:

20 MB extra RAM

during the process.

Why do we care?

Suppose an edge device has:

512 MB RAM

A model needing:

20 MB

is fine.

A model needing:

2 GB

cannot run.

4. What is Training Time?

9.44 sec

This is how long it took to learn the model.

For Edge AI:

Training usually happens:

Cloud
↓
Train Model
↓
Deploy Model

So training time is less important than inference time.

Which metric is most important?

For Edge AI:

Most Important

- Model Size

- Inference Time

- RAM Usage

Less Important

- Training Time

because training is done once.

RAM's job

RAM stores:

- The dataframe df

- Variables

- Libraries (Pandas, NumPy, Scikit-Learn)

- The trained model

# CPU's job

- The CPU actually performs the calculations.

Read data from RAM

↓

Perform mathematical operations

↓

Build decision trees

↓

Store results back into RAM

CPU

Computes:

Which feature to split?
Where to split?
Calculate Gini impurity
Build tree nodes
Repeat hundreds of times

Model size has nothing to do with your Python code size.

How much memory is needed to store the learned model after training.

DAY 3: JUNE 3

Installed Gazebo a simulation tool

Learn:

- World

- Model

- Link

- Joint

- Sensor

# 1. World

- A World is the entire simulation environment.

- Example:

- World
 ├── Ground
 ├── Robot
 ├── Human Actor
 ├── Light
 └── Camera

2. Model

- A Model is an object inside the world.

- Examples:

- Car
Robot
Human Actor
Sphere
Cube

# 3. Link

- A Link is a rigid body.

- Example:

- Human Leg

Thigh Link
Shank Link
Foot Link

# 4. Joint

- A Joint connects two links

# 5. Sensor

- A sensor produces data.

Small tip

Whenever you want to work with MuJoCo in the future:

cd ~/mujoco_projects
source mujoco_env/bin/activate